Suggested Specification: SL-RGL-E

The SafeLED RGL-E includes numerous technical benefits:

- Elevated Runway Guard Light, yellow light.

- The maximum total power consumption for series version is 51 W, for parallel version 46 W.

- Control and Monitoring capabilities with built-in IQ-functionality and the ASP system.

- The electronic components are encapsulated in waterproof polyurethane.

- Built-in flasher unit, 45-50 times per minute.

- The weight of the light fixture is less than 11 kg.

- Over voltage surge and lightning protection.

- The optical head of the light fixture can be adjusted for maximal light performance.

- The front window is made of flat tempered glass.

- Long life time of light fixture.

- All external parts are made of aluminum alloys.

- All fixings and fastenings are stainless steel.

- The light fixture is designed to allow easy maintenance.

- Frangibility complies with FAA or ICAO/IEC.

- Compatible with the 2A system 4.

- Can be powered by an isolating transformer on a standard AGL primary loop 4.

- The LED light fixture follows the same light intensity curve as a halogen lamp with the different CCR levels 4.

- Automatically dim light intensity to 30% on night level 5.

The light fixture meets the requirements and is in compliance with the

standards:

- ICAO: Annex 14 Volume I Paragraph 5.3.22 Configuration A, High Intensity

- FAA: AC150/5345-46D. L-804 6

- FAA: Engineering Brief No. 67C “Light Sources other than Incandescent and Xenon for Airport Lighting and Obstruction Lighting Fixtures”

- IEC 61827

- STAC

For more information, contact ADB Safegate regarding compliance with other standards.