Suggested Specification: SL-TE-E, SL-TH-E

The SafeLED TE-E and TH-E lights include numerous technical benefits:

- Taxiway elevated edge omnidirectional blue LED light.

- Taxiway elevated edge omnidirectional for intermediate holding position yellow LED light.

- The electronic components are encapsulated in waterproof polyurethane.

- Over voltage surge and lightning protection.

- The optical head of the fitting can be adjusted for maximal light performance.

- The optical dome is made of glass.

- Can be powered by an isolating transformer on a standard AGL primary loop or a parallel system.

- CCR can detect LED failure due to fail open technology (on series circuit).

- The LED fitting follows the same light intensity curve as a halogen lamp with the different CCR levels (on series circuit).

- Long LED life time depending on operation.

- All external parts are made of anodized aluminum alloy casting.

- All fixings and fastenings are stainless steel.

- The weight of the fitting is less than 2.7 kg.

- The fitting is designed to allow easy maintenance.

- Supplied as an option with FAA connector.

- Frangibility complies with FAA.

- The total power consumption is lower than 7.5 W.

- Acts as a halogen lamp and does not pulsate due to resistive load.

- Backwards compatible with halogen lamps.

The light is in compliance with standards:

- ICAO: Annex 14 Volume I Paragraphs 5.3.17.

- FAA: AC150/5345-46D. L-861T.

- FAA: Engineering Brief No 67C: Light Sources other than Incandescent and Xenon for Airport Lighting and Obstruction Lighting Fixtures.

- IEC 61827.

- NATO: STANAG 3316.

- STAC.

- MOS 139: Paragraphs 9, 13, 22, 2.