Suggested Specification: INL-RC

- The runway center line inset light shall be bi-directional high intensity complying with ICAO recommendations in Annex 14, Volume I, paragraph 5.3.12, with FAA L-850A standards, and STANAG 3316 standards, CAP168 and British Standards BS 3224.

- It shall be fitted with one or two 6.6 Amps halogen pre-focused dichroic reflector lamps not exceeding 45 Watts. Lamp life shall be at full intensity greater than 1,500 hours for 45 W.

- Its design shall allow separate lighting in both approach direction.

- All external parts shall be made of anodized tempered aluminum

- alloy casting. All fixings and fastenings shall be stainless steel.

- It shall have a maximum outer diameter of 203 mm (8″) and its projection shall not exceed 12.7 mm (1/2″).

- It must be able to be installed directly on an 8″ shallow base, or by means of adapter on a FAA L-868B deep base or a seating ring.

- It will be design to allow easy maintenance.

- The prisms shall not be sealed.

- The filters shall be dichroic.

- The fittings in this model range share many of the same components.

- No internal adjustment shall be needed.

- The weight of the fitting shall be lower than 2.8 kg.